WMZ11B Series

Intelligent Thermistor/Varistor智能复合热、压敏电阻器

- For Preheating (Thermal & Zero Power Dissipation)

用于预热启动(零温升、零功耗)

■ GENERAL 概述

WMZ11B Series Intelligent Thermistor/varistor is designed specially for preheating of the high property load such as Toshiba Ballast / CFL ( zero thermal / zero power dissipation ) .

WMZ11B Series is a kind of complex components, which includes a thermistor Rt and a varistor Rv in series. When the voltage of the circuit is higher than Rv’s varistor voltage at starting, Rv is at short state. So the process of preheating is finished almost by Rt independently. After the tube lights and works normally, the Rv’s voltage will decrease quickly to the level under its varistor voltage, then Rv will change into high-resistance state. So Rt will be at open state and the preheating which is zero thermal/zero power dissipation will be realized.

WMZ11B系列智能复合热、压敏电阻器是因应东芝等用户的特别要求，专为高端电子镇流器/节能灯设计的一种零功耗、零温升型预热启动元件。

WMZ11B系列是一种由PTC热敏电阻器Rt和压敏电阻器Rv串联而成的复合元件。启动时，电压高于Rv压敏电压，Rv处于导通状态，其预热启动过程基本上是由Rt来独立完成的。灯管启动点亮处于正常工作状态后，Rv端电压迅速降低到其压敏电压以下，Rv跃居高阻态，断开Rt，从而使零功耗，零温升预热启动得以实现。

■ PART NUMBERING 产品编号

| WMZ11B | - | 105 | S | 102 | N | U | U | B | 10 | - | 5D | 121K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

⑴                            ⑵                     ⑶     ⑷

⑴ Series系列

⑵ Spec. of PTC Thermistor ( Similar as WMZ11A Series )

PTC热敏电阻规格参数（与WMZ11A系列类似）

⑶ Size of Varistor 压敏电阻尺寸:  5D -Φ5  7D -Φ7  10D - Φ10

⑷ Varistor Vol. & Tol. of Varistor压敏电阻的压敏电压及其允差范围：

820 - 82V  101 - 100V  121 - 120V  151 - 150V    K - ±10%  J - ±5%

■ SPECIFICATION S TABLE规格表

| No. 序号 | Part No. 料 号 | Rated Zero Power Resistance 额定零功率 电阻 RN (Ω) | Curie Temperature 居里温度 TC (℃) | ## Varistor # Voltage 压敏电压 VV (V) （at 1mA） | Max. Permissible Repetitive Turn Over Current 最大可重复 通断电流 IMax ( mA ) | Dimensions (mm) 外形尺寸 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DMax | TMax | d±0.05 | F±1 |
| 1 | WMZ11B -50S681NS-5D101K | 680±30% | 50±7 | 100±10% 120±10% 150±10% 180±10% 200±10% | 200 | 6.5 | 6.5 | 0.6 | 5.0 |
| 2 | WMZ11B -50S102NS-5D121K | 1000±30% |  |  |  |  |  |  |  |
| 3 | WMZ11B -50S152NS-5D151K | 1500±30% |  |  |  |  |  |  |  |
| 4 | WMZ11B -50S222NS-5D181K | 2200±30% |  |  |  |  |  |  |  |
| 5 | WMZ11B -50S332NS-5D201K | 3300±30% |  |  |  |  |  |  |  |
| 6 | WMZ11B -75S471NS-5D101K //////201K | 470±30% | 75±7 | 100±10% 120±10% 150±10% 180±10% 200±10% | 200 | 6.5 | 6.5 | 0.6 | 5.0 |
| 7 | WMZ11B -75S681NS-5D121K | 680±30% |  |  |  |  |  |  |  |
| 8 | WMZ11B -75S102NS-5D151K | 1000±30% |  |  |  |  |  |  |  |
| 9 | WMZ11B -75S152NS-5D181K | 1500±30% |  |  |  |  |  |  |  |
| 10 | WMZ11B -75S222NS-5D201K | 2200±30% |  |  |  |  |  |  |  |
| 11 | WMZ11B -105S471NS-5D101K | 470±30% | 105±7 | 100±10% 120±10% 150±10% 180±10% 200±10% | 200 | 6.5 | 6.5 | 0.6 | 5.0 |
| 12 | WMZ11B -105S681NS-5D121K | 680±30% |  |  |  |  |  |  |  |
| 13 | WMZ11B -105S102NS-5D151K | 1000±30% |  |  |  |  |  |  |  |
| 14 | WMZ11B -105S152NS-5D181K | 1500±30% |  |  |  |  |  |  |  |
| 15 | WMZ11B -105S222NS-5D201K | 2200±30% |  |  |  |  |  |  |  |
| 16 | WMZ11B -105S471NS-7D201K | 470±30% | 105±7 | 200±10% 270±10% 300±10% 330±10% 390±10% | 200 | 8.5 | 6.5 | 0.6 | 5.0 |
| 17 | WMZ11B -105S681NS-7D271K | 680±30% |  |  |  |  |  |  |  |
| 18 | WMZ11B -105S102NS-7D301K | 1000±30% |  |  |  |  |  |  |  |
| 19 | WMZ11B -105S152NS-7D331K | 1500±30% |  |  |  |  |  |  |  |
| 20 | WMZ11B -105S222NS-7D391K | 2200±30% |  |  |  |  |  |  |  |
| 21 | WMZ11B -75L471NS-5D101K | 470±30% | 75±7 | 100±10% 120±10% 150±10% 180±10% 200±10% | 300 | 6.5 | 6.5 | 0.6 | 5.0 |
| 22 | WMZ11B -75L681NS-5D121K | 680±30% |  |  |  |  |  |  |  |
| 23 | WMZ11B -75L102NS-5D151K | 1000±30% |  |  |  |  |  |  |  |
| 24 | WMZ11B -75L152NS-5D181K | 1500±30% |  |  |  |  |  |  |  |
| 25 | WMZ11B -75L222NS-5D201K | 2200±30% |  |  |  |  |  |  |  |
| 26 | WMZ11B -85L471NS-5D101K | 470±30% | 85±7 | 100±10% 120±10% 150±10% 180±10% 200±10% | 300 | 6.5 | 6.5 | 0.6 | 5.0 |
| 27 | WMZ11B -85L681NS-5D121K | 680±30% |  |  |  |  |  |  |  |
| 28 | WMZ11B -85L102NS-5D151K | 1000±30% |  |  |  |  |  |  |  |
| 29 | WMZ11B -85L152NS-5D181K | 1500±30% |  |  |  |  |  |  |  |
| 30 | WMZ11B -85L222NS-5D201K | 2200±30% |  |  |  |  |  |  |  |
| 31 | WMZ11B -85L471NS-7D151K | 470±30% | 85±7 | 150±10% 200±10% 240±10% 270±10% 330±10% | 300 | 8.5 | 6.5 | 0.6 | 5.0 |
| 32 | WMZ11B -85L681NS-7D201K | 680±30% |  |  |  |  |  |  |  |
| 33 | WMZ11B -85L102NS-7D241K | 1000±30% |  |  |  |  |  |  |  |
| 34 | WMZ11B -85L152NS-7D271K | 1500±30% |  |  |  |  |  |  |  |
| 35 | WMZ11B -85L222NS-7D331K | 2200±30% |  |  |  |  |  |  |  |
| 36 | WMZ11B -75A471NS-7D151K | 470±30% | 75±7 | 150±10% 200±10% 240±10% 270±10% 330±10% | 400 | 8.5 | 6.5 | 0.6 | 5.0 |
| 37 | WMZ11B -75A681NS-7D201K | 680±30% |  |  |  |  |  |  |  |
| 38 | WMZ11B -75A102NS-7D241K | 1000±30% |  |  |  |  |  |  |  |
| 39 | WMZ11B -75A152NS-7D271K | 1500±30% |  |  |  |  |  |  |  |
| 40 | WMZ11B -75A222NS-7D331K | 2200±30% |  |  |  |  |  |  |  |
| 41 | WMZ11B -75A471NS-10D201K | 470±30% | 75±7 | 200±10% 240±10% 270±10% 330±10% | 400 | 11.5 | 6.5 | 0.6 | 7.5 |
| 42 | WMZ11B -75A681NS-10D241K | 680±30% |  |  |  |  |  |  |  |
| 43 | WMZ11B -75A102NS-10D271K | 1000±30% |  |  |  |  |  |  |  |
| 44 | WMZ11B -75A152NS-10D331K | 1500±30% |  |  |  |  |  |  |  |
| 45 | WMZ11B -105A471NS-7D201K | 470±30% | 105±7 | 200±10% 240±10% 270±10% 330±10% | 400 | 8.5 | 6.5 | 0.6 | 5.0 |
| 46 | WMZ11B -105A681NS-7D241K | 680±30% |  |  |  |  |  |  |  |
| 47 | WMZ11B -105A102NS-7D271K | 1000±30% |  |  |  |  |  |  |  |
| 48 | WMZ11B -105A152NS-7D331K | 1500±30% |  |  |  |  |  |  |  |
| 49 | WMZ11B -85B331NS-7D201K | 330±30% | 85±7 | 200±10% 240±10% 270±10% 330±10% | 800 | 8.5 | 6.5 | 0.6 | 7.5 |
| 50 | WMZ11B -85B471NS-7D241K | 470±30% |  |  |  |  |  |  |  |
| 51 | WMZ11B -85B681NS-7D271K | 680±30% |  |  |  |  |  |  |  |
| 52 | WMZ11B -85B102NS-7D331K | 1000±30% |  |  |  |  |  |  |  |
| 53 | WMZ11B -85B331NS-10D201K | 330±30% | 85±7 | 200±10% 240±10% 270±10% 330±10% | 800 | 11.5 | 6.5 | 0.6 | 7.5 |
| 54 | WMZ11B -85B471NS-10D241K | 470±30% |  |  |  |  |  |  |  |
| 55 | WMZ11B -85B681NS-10D271K | 680±30% |  |  |  |  |  |  |  |
| 56 | WMZ11B -85B102NS-10D331K | 1000±30% |  |  |  |  |  |  |  |